Republic of thePhilippines

PARTIDO DEVELOPMENT ADMINISTRATION

Created Under R.A. 7820

Tigaon, Camarines Sur

____________ , 20 _____

PRICE QUOTATION

_______________________

_______________________

Sir/Madam:

Please quote your prices of the following articles which this office desire to purchase for use in rehabilitation of 200KVA “Johndeere” genset in Caramoan Water Treatment Plant.

| ITEM # | PARTICULARS | QTY. | UNIT | UNIT COST | TOTAL COST |
| --- | --- | --- | --- | --- | --- |
| 1 | SUPPLY OF LABOR AND MATERIALS FOR REHABILITATION OF 200 KVA, 440 VAC, 3 PHASE “JOHNDEERE” GENSET in Caramoan WTP. Scope of Work: ITEM 1: General overhauling of main engine brand/model: Johndeere/6068HF150 - Dismantling of engine parts & components - Clean/inspection of engine parts/components. - Hydrotest/crack test engine block. - Check/inspection of cyclinder head assy., piston connecting rod, cam shaft, crankshaft, intake & exhaust valve, radiator assy., water pump, etc. - Calibration/Testing of fuel injection pump & injectors. - Replacement of damaged/defective parts, i.e. overhauling gasket, main bearing, connecting rod bearing, piston ring, rocker arm, etc. - Machine shop works of engine/engine parts - Other related works, fabrication/installation engine component & attachment - Re-assy of engine component parts - Test run g. set w/out load and w/ load @ 40% & 80% generating capacity. ITEM 2: Rehab/servicing of alternator assy. Brand/model: Leroy SOmer/200 KVA - Dismantling of alternator assy. & component parts including elec. Wiring connections - Cleaning/inspection of main stator, rotor, exciter windings including rectifier - Rewinding, re-insulation, re-varnish, baking & oven-dry stator, rotor & exciter winding - Replacement of damaged/defective parts of alternator assy. & components parts. - Re-assy. Of alternator assy., re-install control panel assy. including electrical wiring conn. - Test run g. set with & without load & check operational performance. | 1 | lot |  |  |

Dealer:		______________________________________

TIN: VAT#:	______________________________________ NON-VAT#:_____________________

Address:	______________________________________

Telephone No.:	______________________________________

Contact Person:	______________________________________

Canvasser:	______________________________________

Sir/Madam:

I have the honor to offer my prices for each of the above articles as indicated after each item hereon.

Very truly yours

______________________